Q36. MQTT vs HTTP Comparison

Answer : MQTT vs HTTP Comparison

| Aspect | MQTT | HTTP |
| --- | --- | --- |
| Architecture pattern | Publish/subscribe via a central broker; devices don't talk directly to each other | Request/response (client-server); each interaction needs a direct connection to a server |
| Data transfer model | Lightweight, binary-friendly, persistent TCP connection; messages pushed as events occur | Text-based (headers, verbose), typically opens a new connection per request (unless keep-alive) |
| Power consumption | Low — one persistent, lightweight connection; minimal overhead per message | Higher — repeated connection setup/teardown (TCP + TLS handshakes) drains battery faster |
| Latency | Low — near real-time push, no polling needed | Higher — depends on polling frequency; each request has handshake overhead |
| Use case suitability for IoT | Excellent — designed for constrained devices, unreliable networks, many-to-many communication | Poor to moderate — better suited for occasional, request-driven interactions (e.g., firmware updates, dashboards) |
| Security considerations | TLS support, username/password, client certificates; broker becomes a single point of trust/failure — must be hardened | HTTPS/TLS is mature and widely supported; but overhead makes frequent secure calls costly on constrained devices |

## Recommendation: MQTT

For a 1000-node smart agriculture deployment, MQTT is the clear choice:

- Battery life matters. Field sensors (soil moisture, temperature, humidity) are often solar- or battery-powered and deployed in remote areas with infrequent maintenance access. MQTT's low overhead directly translates to longer battery life.

- Scale favors pub/sub. With 1000 nodes, a pure request/response model creates heavy connection overhead on the server side. MQTT's broker can efficiently fan out data to multiple subscribers (e.g., irrigation controller, analytics dashboard, mobile app) from a single sensor publish — no need for each sensor to know who's consuming its data.

- Unreliable connectivity. Rural/agricultural sites often have spotty cellular or LPWAN connectivity. MQTT supports QoS levels (0/1/2) and "last will" messages, so you get delivery guarantees and offline-device detection — hard to replicate cleanly over HTTP.

- Real-time responsiveness. Alerts like "soil moisture critically low" or "frost warning" need to reach an irrigation actuator quickly. MQTT's event-driven push avoids the latency of polling intervals.

- Bandwidth costs. If some nodes use cellular data (paid per MB), MQTT's compact binary framing reduces data costs meaningfully at scale compared to HTTP's verbose headers.

Where HTTP still has a role: Use HTTP/HTTPS for less frequent, heavier operations — e.g., OTA firmware updates, historical data queries from a cloud dashboard, or third-party API integrations (weather services, market price feeds). Many real deployments use MQTT for sensor-to-cloud telemetry and HTTP/REST for management and analytics layers — a hybrid approach rather than an either/or.

### Q37. Wi-Fi Security for IoT

ANSWER: Wi-Fi Security in IoT: Attack Vectors, Best Practices, and Hardcoded Credentials

IoT devices connecting over Wi-Fi face several persistent threats. A major flaw in WPA2, KRACK (Key Reinstallation Attack), exploits the four-way handshake and could theoretically affect nearly every WPA2 device, though unpatched IoT devices such as cameras and smart-home gear remain particularly at risk since they rarely receive firmware updates. Evil twin attacks compound this risk: attackers set up a rogue access point sharing the same SSID as a legitimate one, tricking clients into connecting so traffic can be intercepted via man-in-the-middle techniques. Because many IoT devices auto-connect to known SSIDs without robust certificate checks, they are especially susceptible to this technique.

For securing ESP32-based deployments, developers should move beyond WPA2 where possible. WPA3, the latest Wi-Fi Alliance standard, offers stronger protections against many known attacks, though WPA2 still dominates most home and business networks during this transition period. Certificate pinning further hardens ESP32 connections by validating the server's TLS certificate against a known, embedded copy rather than trusting any certificate authority, preventing spoofed access points from silently intercepting encrypted traffic.

Hardcoding Wi-Fi or API credentials directly into ESP32 firmware is dangerous because compiled binaries can be extracted and reverse-engineered, exposing plaintext secrets. Since IoT firmware is rarely updated after deployment, a leaked credential can compromise an entire fleet of devices simultaneously with no easy remediation path, unlike a server-side password reset.

Sources:

- SecurityScientist.net – 12 Questions and Answers About KRACK WPA2 WiFi Vulnerability

- CyberSecura (Medium) – WPA/WPA2 Cracking, PMKID, Evil Twin: Overview of Attacks and Threats to Wi-Fi

### Q38. LoRa & LPWAN – Long Range IoT

Answer :  LoRa & LPWAN – Long Range IoT

LPWAN (Low Power Wide Area Network) refers to wireless technologies designed to connect battery-powered sensors over large distances while consuming minimal energy — trading high bandwidth for range and endurance, unlike Wi-Fi or cellular networks.

LoRa achieves this using Chirp Spread Spectrum (CSS) modulation, where the spreading factor (SF), ranging from SF7 to SF12, controls how much a signal is "stretched" in time. Higher spreading factors improve receiver sensitivity and range but increase transmission time and power consumption, while lower spreading factors give faster data rates over shorter distances. Networks often use Adaptive Data Rate (ADR) to balance this trade-off automatically, letting distant nodes use high SF for reach while nearby nodes use low SF to save power and airtime.

LoRa vs Zigbee vs Z-Wave: LoRa offers the longest range (kilometers) but low data rates and star topology via gateways; Zigbee provides mesh networking with short range (~10-100m) but higher data throughput, suited for dense indoor sensor networks; Z-Wave is similar to Zigbee but proprietary, optimized for home automation with limited node capacity (~232 devices).

Real-world India example: In Kashmir, LoRaWAN technology has been deployed to transform traditional apple farming into precision agriculture, addressing climate monitoring and resource utilization challenges, enabling farmers to make data-driven irrigation and pest-control decisions across large orchards.

Q39. (Theory) What is an IoT gateway? Draw a simple diagram showing the relationship between: IoT Sensor Nodes → Gateway → Cloud → User Application. What protocols are typically used at each layer? (2 marks)

Answer: An IoT Gateway is a device (hardware/software) that sits between low-power sensor nodes and the cloud/internet. It aggregates, filters, and translates data from multiple sensors (often using short-range/low-power protocols) into internet-friendly protocols before forwarding it to the cloud. It also handles tasks like local processing (edge computing), protocol translation, security (encryption/authentication), and buffering data when connectivity is lost.

Typical protocols at each layer:

- Sensor Nodes → Gateway: Short-range, low-power protocols like Zigbee, BLE, or LoRa — suited for battery-constrained devices.

- Gateway → Cloud: Internet-friendly protocols like MQTT, HTTP/HTTPS, or CoAP — the gateway translates local sensor data into these formats.

- Cloud → User Application: REST APIs or WebSockets — used to deliver processed data to dashboards or mobile apps in real time.

Q40. (Theory) Explain QoS (Quality of Service) levels in MQTT: QoS 0 (At most once), QoS 1 (At least once), QoS 2 (Exactly once). In an IoT water level monitoring system, which QoS level would you choose and why? (2 marks)

Answer : MQTT QoS Levels

QoS 0 – At most once The message is sent once with no acknowledgment and no guarantee of delivery. It's "fire and forget" — fastest and lowest overhead, but messages can be lost if the connection drops.

QoS 1 – At least once The sender keeps resending the message until it receives an acknowledgment (PUBACK) from the receiver. This guarantees delivery, but the same message may be delivered more than once (duplicates possible) if the ACK itself is lost.

QoS 2 – Exactly once Uses a four-part handshake (PUBLISH → PUBREC → PUBREL → PUBCOMP) to guarantee the message is delivered exactly one time, with no loss and no duplication. This is the most reliable but also the slowest and most resource-intensive, since it requires more round trips and message storage.

## Recommendation for a Water Level Monitoring System

QoS 1 (At least once) is the most practical choice.

Why:

- Reliability matters, but exact-once isn't critical. Water level data is often used for alerts (e.g., tank overflow, low water warning) — missing a reading (as could happen with QoS 0) is riskier than receiving a duplicate reading. QoS 1 ensures the alert/data actually reaches the server.

- Duplicates are harmless here. If the same water-level reading arrives twice, it doesn't cause harm — you can just overwrite or average it. This makes QoS 2's strict "exactly once" guarantee unnecessary overhead.

- Resource efficiency for constrained devices. QoS 2's four-way handshake consumes more bandwidth, memory, and processing — a burden for battery-powered ESP32/sensor nodes. QoS 1 strikes a good balance between reliability and resource usage.

When QoS 2 might be justified: If the system triggers a critical, one-time actuation (e.g., automatically shutting off a pump when water reaches a threshold) where a duplicate command could cause equipment damage, QoS 2 would be worth the extra overhead for that specific message type — while routine sensor readings could still use QoS 1.

Ans 42 CODE-

Start

Initialize ESP32

Initialize Soil Moisture Sensor

Initialize LDR Sensor

Initialize Relay (Pump)

Initialize Push Button

Initialize LCD

Connect to Wi-Fi

Loop

Read Soil Moisture Value

Read LDR Value

If Manual Button is Pressed

Turn Pump ON

Wait 5 Minutes

Turn Pump OFF

Else

If Soil is Dry

If LDR Detects Rain (Low Light)

Pump OFF

Else

Pump ON

End If

Else

Pump OFF

End If

End If

Display Moisture Percentage

Display Pump Status

Display Last Watered Time

Upload Data to Google Sheets

Wait 30 Minutes

Repeat

FLOWCHART-

START

│

▼

Initialize ESP32

│

▼

Read Soil Moisture Sensor

│

▼

Read LDR Sensor

│

▼

Is Manual Button Pressed?

│          	│

Yes         	No

│          	│

▼          	▼

Turn Pump ON 	Check Soil Moisture

for 5 Minutes      	│

▼

Is Soil Dry?

│     	│

No     	Yes

│      	│

▼      	▼

Pump OFF   Is It Dark

(Possible Rain)?

│      	│

Yes     	No

│       	│

▼       	▼

Pump OFF 	Pump ON

│       	│

└──────┬────┘

▼

Display Moisture, Mode &

Last Watered Time on LCD

│

▼

Send Data to Google Sheets

│

▼

Repeat Loop

Ans 41.                     START

│

▼

Initialize ESP32

│

▼

Initialize MQ-2, DHT11,

RGB LED, Buzzer & Bluetooth

│

▼

Read MQ-2 Gas Sensor Value

│

▼

Read Temperature & Humidity

from DHT11 Sensor

│

▼

Compare Gas Value with

Predefined Thresholds

│

┌───────────┼───────────┐

▼       	▼       	▼

SAFE      MODERATE 	DANGER

│       	│       	│

Green LED ON   Yellow LED ON  Red LED ON

Short Beep     Medium Beep	Long Beep

│       	│       	│

└───────────┼───────────┘

▼

Display Data on Serial Monitor

(CSV Format with Timestamp)

│

▼

Send Alert to Mobile via

Bluetooth (if Warning/Danger)

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Code-

StartInitialize ESP32

Initialize MQ-2 Sensor

Initialize DHT11 Sensor

Initialize RGB LED

Initialize Buzzer

Initialize Bluetooth

Loop Forever

Read MQ-2 Gas Value

Read Temperature

Read Humidity

If Gas Value < Safe Threshold

Turn ON Green LED

Turn OFF Yellow & Red LED 	Buzzer OFF

Severity = SAFE

Else If Gas Value < Danger Threshold

Turn ON Yellow LED

Turn OFF Green & Red LED

Play Medium Beep

Severity = MODERATE

Else

Turn ON Red LED

Turn OFF Green & Yellow LED

Play Long Beep

Severity = DANGER

End If

Print Timestamp

Print Gas Value

Print Temperature

Print Humidity

Print Severity

If Severity is MODERATE or DANGER

Send Bluetooth Alert

End I

Wait 2 Seconds

Repeat

Sample serial monitor output-

Timestamp,MQ2 Value,DHT11 Temp,DHT11 Humidity,Severity

00:01,220 ppm,23.4°C,49%,SAFE

00:02,280 ppm,23.5°C,50%,SAFE

00:03,420 ppm,23.6°C,49%,MODERATE 00:04,710 ppm,23.8°C,48%,DANGER

Sample mobile alert-

SAFE Gas: 210 ppm

Temperature: 22.8°C

Humidity: 51%

Status: SAFE

WARNING Moderate Smoke Detected

Gas: 430 ppm

Check Fireplace

DANGER

Smoke Level Critical

Gas: 765 ppm

Open Windows

Leave Room if Necessary

Ans 43.

START

│

▼

Initialize ESP32

│

▼

Initialize Keypad, LCD/OLED,

Bluetooth and Servo Motor

│

▼

User Enters 4-digit PIN

│

▼

Is PIN Correct?

│      	│

No     	Yes

│      	│

▼      	▼

Display Wrong   Generate

PIN     	6-digit OTP

│      	│

▼      	▼

Log Failure   Send OTP via

Bluetooth

│

▼

Start 30-sec Timer

│

▼

User Enters OTP

│

┌─────────┴─────────┐

▼               	▼

OTP Correct & Time   	Wrong OTP/

≤30 Seconds      	Timer Expired

│               	│

▼               	▼

Rotate Servo 90°    	Access Denied

Display Access      	Log Failure

Granted

│

▼

Log Success

│

▼

Repeat

Start

Initialize ESP32

Initialize Keypad

Initialize LCD

Initialize Bluetooth

Initialize Servo

Loop

Read 4-digit PIN

If PIN is Correct

Generate Random 6-digit OTP

Send OTP via Bluetooth

Start 30-second Timer

Read OTP from User

If OTP is Correct AND Time < 30 Seconds

Rotate Servo to 90 Degrees

Display "Access Granted"

Log Success to Serial Monitor

Else

Display "Access Denied"

Log Failure to Serial Monitor

End If

Else

Display "Wrong PIN"

Log Failure to Serial Monitor

End If

Repeat

Sample serial monitor output

Time,PIN,OTP,Status

10:00:15,1234,654321,ACCESS GRANTED

10:05:40,1234,125678,WRONG OTP

10:10:12,9999,------,WRONG PIN

10:15:50,1234,Expired,OTP TIMEOUT

Sample mobile alert

SMART DOOR LOCK

Your OTP is: 654321

Valid for 30 seconds.

Ans 44.                  START

│

▼

Initialize ESP32

│

▼

Initialize PIR Sensor, Buzzer,

LEDs and Telegram

│

▼

Read Current Time Window

│

▼

Is Time Within Security Hours?

│         	│

No        	Yes

│         	│

▼         	▼

Keep Monitoring   Read PIR Sensor

│

▼

Motion Detected?

│     	│

No    	Yes

│     	│

▼         ▼

Continue Loop   Turn ON LED 1

Play Warning Beep

│

▼

Turn ON LED 2

Play Alarm Beep

│

▼

Turn ON LED 3

Play Urgent Beep

│

▼

Send Telegram Alert

Log Event with Time

│

▼

Repeat Loop

Code- Start

Initialize ESP32

Initialize PIR Sensor

Initialize Buzzer

Initialize Three LEDs

Initialize Telegram

Set Security Time Window

Loop

Read Current Time

If Current Time is Within Security Hours

Read PIR Sensor

If Motion Detected

Turn ON LED1

Play Warning Beep

Turn ON LED2

Play Alarm Beep

Turn ON LED3

Play Urgent Beep

Send Telegram Alert

Log Motion Event with Timestamp

Else

Turn OFF LEDs

Turn OFF Buzzer

End If

End If

Repeat

Sample serial monitor- Time,Motion,Alert Level,Status

22:01:15,Detected,Warning,LED1 ON

22:01:18,Detected,Alarm,LED2 ON

22:01:22,Detected,Urgent,LED3 ON

22:01:25,Detected,Telegram Sent

22:05:40,No Motion,System Monitoring

Sample telegram output

🚨 SECURITY ALERT 🚨

Motion Detected at Shop

Time: 22:01:25

Alert Level: URGENT

Please Check Immediately.

Ans 45.                  START

│

▼

Initialize ESP32

│

▼

Initialize DHT11, LDR, LCD,

Heater LED, Fan LED, Grow LED

│

▼

Read Temperature, Humidity & LDR

│

▼

Calculate Running Average of LDR

(Last 10 Readings)

│

▼

Is Temperature < 18°C?

│          	│

Yes         	No

│          	│

▼          	▼

Heater ON       Is Temperature > 22°C?

│        	│

Yes       	No

│        	│

▼        	▼

Fan ON  	Heater & Fan OFF

│

▼

Is Humidity Between 60–70%?

│

▼

Check Light Intensity (LDR)

│

▼

Is Light Sufficient for 12 Hours?

│             	│

No            	Yes

│             	│

▼ 	            ▼

Grow Light ON 	Grow Light OFF

│

▼

Display Values on LCD

Publish Data to MQTT

│

▼

Wait 3 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32

Initialize DHT11

Initialize LDR

Initialize LCD

Initialize Heater LED

Initialize Fan LED

Initialize Grow Light LED

Initialize MQTT

Loop

Read Temperature

Read Humidity

Read LDR Value

Calculate Running Average of Last 10 LDR Readings

If Temperature < 18

Turn ON Heater

Turn OFF Fan

Else If Temperature > 22

Turn ON Fan

Turn OFF Heater

Else

Turn OFF Heater

Turn OFF Fan

End If

If Light Level is Low

Turn ON Grow Light

Else

Turn OFF Grow Light

End If

Display Temperature

Display Humidity

Display Light Level

Publish Data to MQTT

Wait 3 Seconds

Repeat

Sample LCD display

Temp : 20°C

Humidity : 65%

Light : Normal

Heater : OFF

Fan : OFF

Grow Light : OFF

Sample serial monitor output

Time,Temp,Humidity,Light,Heater,Fan,GrowLight

10:00:00,19.5,64,420,OFF,OFF,OFF

10:00:03,17.2,63,300,ON,OFF,ON

10:00:06,22.8,66,520,OFF,ON,OFF

10:00:09,20.1,65,410,OFF,OFF,OFF

Ans 46.                  START

│

▼

Initialize ESP32

│

▼

Initialize HC-SR04, LEDs,

Buzzer, Wi-Fi & Dashboard

│

▼

Read Ultrasonic Distance

│

▼

Calculate Moving Average (10 Readings)

│

▼

Is Distance > 30 cm?

│         	│

Yes        	No

│         	│

▼         	▼

NORMAL Level   Is Distance > 15 cm?

│        	│

Yes 	      No

│        	│

▼        	▼

WARNING Level   CRITICAL Level

│        	│

Yellow LED + Medium Beep   Red LED + Fast Beep

│        	│

└──────┬─────┘

▼

Send Alert via Wi-Fi/Bluetooth

│

▼

Update Live Chart on Dashboard

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32

Initialize HC-SR04

Initialize LEDs

Initialize Buzzer

Initialize Wi-Fi

Initialize Dashboard

Loop

Read Distance

Calculate Moving Average

If Distance > 30

Turn ON Green LED

Turn OFF Yellow LED

Turn OFF Red LED

Buzzer OFF

Level = NORMAL

Else If Distance > 15

Turn ON Yellow LED

Turn OFF Green LED

Turn OFF Red LED

Medium Beep

Level = WARNING

Else

Turn ON Red LED

Turn OFF Green LED

Turn OFF Yellow LED

Fast Beep

Level = CRITICAL

End If

Send Alert Message

Update Dashboard Graph

Print Distance and Level

Wait 2 Seconds

Repeat

Sample seriaql monitor

Time,Distance(cm),Water Level,Status

10:00:00,42,NORMAL,SAFE

10:00:02,28,WARNING,ALERT

10:00:04,12,CRITICAL,DANGER

10:00:06,10,CRITICAL,DANGER

Ans 47.                  START

│

▼

Initialize ESP32

│

▼

Initialize MAX30102/LDR, LCD,

LED and Data Log File

│

▼

Read Heart Rate & SpO₂

│

▼

Calculate 5-Reading Rolling Average

│

▼

Display Values on LCD

│

▼

Is Heart Rate <50 or >120?

│

┌────────┴────────┐

▼             	▼

Yes           	No

│             	│

▼             	▼

Turn ON Red LED   Is SpO₂ <94%?

│

┌────────┴────────┐

▼             	▼

Yes           	No

│             	│

▼             	▼

Turn ON Red LED	LED OFF

│

▼

Save Data to health_log.csv

│

▼

Wait 2 Seconds

│

▼

Repeat Loop

Start

Initialize ESP32 Initialize MAX30102 Sensor

Initialize LCD

Initialize Red LED

Initialize Data Log File

Loop

Read Heart Rate

Read SpO2

Calculate 5 Reading Rolling Average

Display Heart Rate on LCD

Display SpO2 on LCD

If Heart Rate < 50 OR Heart Rate > 120

Turn ON Red LED

Else If SpO2 < 94

Turn ON Red LED

Else

Turn OFF Red LED

End If

Save Heart Rate and SpO2 to health_log.csv

Wait 2 Seconds

Repeat

Sample serial monitor output

Time,Heart Rate,SpO2,Status

10:00:00,76,98,NORMAL

10:00:02,82,97,NORMAL

10:00:04,124,96,HIGH HEART RATE

10:00:06,78,92,LOW SPO2

10:00:08,79,98,NORMAL

Ans 48

Street Lighting System

Sensors Needed

- LDR Sensor

- PIR Motion Sensor

Microcontroller

- ESP32

Connectivity

- Wi-Fi (MQTT)

Data Flow

LDR + PIR Sensor → ESP32 → MQTT → Cloud → Dashboard

Approx. cost

ESP32 - ₹500

LDR - ₹50

PIR - ₹120

Relay - ₹100

LED - ₹50

Total ≈ ₹820

Start

Initialize ESP32

Initialize LDR

Initialize PIR

Initialize Relay

Loop

Read LDR

Read PIR

If Light is Low AND Motion Detected

Turn ON Street Light

Else

Turn OFF Street Light

End If

Send Status to Cloud

Repeat

# Air Quality Monitoring System

### Sensors Needed

- MQ-2 Gas Sensor

- DHT11

### Microcontroller

- ESP32

### Connectivity

- Wi-Fi (MQTT)

### Data Flow

MQ2 + DHT11 → ESP32 → MQTT → Cloud → Dashboard

### Approximate Cost

ESP32 - ₹500

MQ-2 - ₹180

DHT11 - ₹120

Total ≈ ₹800

### Pseudo Code

Start

Initialize ESP32

Initialize MQ2

Initialize DHT11

Loop

Read Gas Value

Read Temperature

Read Humidity

If Gas Value is High

Send Alert

End If

Upload Data to Cloud

Repeat

Ans 49.

## Security Vulnerabilities

| Vulnerability | Risk | Solution |
| --- | --- | --- |
| Hardcoded Wi-Fi SSID and Password | Credentials can be stolen if the code is shared. | Store credentials in config.h and exclude it using .gitignore. |
| Using HTTP instead of HTTPS | Data can be intercepted or modified by attackers. | Use HTTPS with SSL/TLS encryption. |
| Public MQTT Broker without Authentication | Anyone can publish or subscribe to messages. | Use MQTT username/password authentication and TLS. |
| No Validation of Incoming Commands | Attackers can send malicious commands to the device. | Validate all received commands before executing them. |
| No Data Encryption | Sensitive data is transmitted in plain text. | Encrypt communication using TLS/SSL. |

## Corrected Code 1 (Store Credentials Securely)

#include "config.h"

const char* ssid=WIFI_SSID;

const char* password=WIFI_PASSWORD;

config.h

#define WIFI_SSID "Your_WiFi_Name"

#define WIFI_PASSWORD "Your_WiFi_Password"

## Corrected Code 2 (Use HTTPS Instead of HTTP)

WiFiClientSecure client;

HTTPClient https;

https.begin(client,"https://example.com/api");

int httpCode=https.GET();

## Corrected Code 3 (Validate Commands)

String cmd=receivedCommand;

if(cmd=="ON"){

digitalWrite(LED,HIGH);

}

else if(cmd=="OFF"){

digitalWrite(LED,LOW);

}

else{

Serial.println("Invalid Command");

}

# Ans 50. Reflection & Learning Portfolio

During this IoT assignment, I learned how different sensors, microcontrollers, and communication technologies work together to build smart systems. I explored various IoT applications such as air quality monitoring, smart door locks, automated plant watering, greenhouse monitoring, flood detection, and healthcare monitoring. While designing these projects, I understood how sensors collect real-world data, how the ESP32 processes that data, and how information is transmitted using Wi-Fi, Bluetooth, HTTP, and MQTT protocols. I also learned the importance of cloud platforms, dashboards, and mobile applications for monitoring and controlling IoT devices remotely.

Another important learning outcome was understanding IoT security. I studied common security risks such as hardcoded passwords, unsecured HTTP communication, public MQTT brokers, and lack of data validation. I learned that secure communication using HTTPS, MQTT authentication, WPA3, and proper credential management are essential for protecting IoT devices from cyber attacks. This has helped me understand that security should always be considered while designing any connected system.

I also gained knowledge about designing algorithms, flowcharts, and system architecture before writing code. Creating flowcharts made it easier to understand the working of each project step by step. I learned how different sensors are selected based on the application, how threshold values are used for decision-making, and how alerts are generated using LEDs, buzzers, Bluetooth, or cloud notifications.

As an Electronics and Communication Engineering student, these IoT concepts are highly useful because they combine embedded systems, communication, networking, and programming. In the future, I would like to improve my skills by building complete hardware prototypes and connecting them to cloud platforms for real-time monitoring.

My next IoT project idea is a Smart Classroom Monitoring System. It would use sensors such as DHT11 for temperature and humidity, an MQ-2 sensor for air quality, an LDR for light intensity, and an ultrasonic sensor for occupancy detection. The ESP32 would collect sensor data and send it to a cloud dashboard using MQTT over Wi-Fi. Teachers and administrators could monitor classroom conditions through a mobile application, receive alerts when environmental conditions become unhealthy, and automatically control lights and fans to save energy. This project would improve student comfort, reduce electricity consumption, and make classrooms smarter and more energy efficient.

Overall, this assignment improved my understanding of IoT system design, sensor integration, wireless communication, cloud connectivity, and security. It also strengthened my problem-solving skills and gave me confidence to work on more advanced IoT projects in the future.