# IoT Assignment (02-07-2026)

Answer:1

I would choose the ESP32 because it has built-in Wi-Fi and Bluetooth, a faster dual-core processor, more RAM, and more GPIO pins than the Arduino UNO. Since the system needs to send sensor data to a mobile application every 5 seconds, internet connectivity is essential. The ESP32 can easily read the soil moisture sensor, control the relay for the water pump, and communicate with cloud services or mobile applications simultaneously.

If internet connectivity were not required, I would choose the Arduino UNO because it is simpler, less expensive, consumes less power in basic applications, and is sufficient for reading soil moisture and controlling the water pump locally.

Conclusion: ESP32 is the better choice when IoT connectivity is needed, while Arduino UNO is suitable for standalone automation.

Answer: 2 For a factory with 500 sensors spread across 2 km, I would choose:

- LoRa for communication between sensors and gateways because it provides long-range coverage with very low power consumption.

- MQTT for sending data from the gateway to the cloud because it is lightweight, reliable, and designed for IoT applications.

- Wi-Fi can be used only at the gateway where internet connectivity is required.

This combination provides long range, scalability, and efficient communication.

Answer: 3

To identify the fault, I would troubleshoot step by step.

- Check the sensor

  - Replace the sensor with a working one.

  - Compare readings.

- Check wiring

  - Inspect loose wires or damaged connections.

  - Ensure correct pin connections.

- Check power supply

  - Measure supply voltage using a multimeter.

  - Verify that the sensor receives the required voltage.

- Check ADC (for analog sensors)

  - Read ADC values directly.

  - Compare with expected voltage levels.

- Check software

  - Verify pin numbers.

  - Check sensor libraries.

  - Print raw values through Serial Monitor.

  - Confirm unit conversions are correct.

By testing one component at a time, the exact source of the problem can be identified.

Answer:4

To allow an ESP32 weather station to operate for six months on battery power, I would make the following improvements:

### By using of Hardware

- Use a high-capacity lithium battery.

- Choose low-power sensors.

- Use an efficient voltage regulator.

- Disconnect unused peripherals.

- Use a solar panel if possible.

### By using of Software

- Enable Deep Sleep Mode between measurements.

- Wake up only when sensor readings are required.

- Reduce Wi-Fi transmission frequency.

- Compress data before transmission.

- Turn off Wi-Fi immediately after uploading data.

- Minimize processor activity.

These methods significantly reduce power consumption and extend battery life.

Answer: 5

I will choose the CAN (Controller Area Network) bus. Because

- Very low communication delay.

- High reliability.

- Excellent error detection and correction.

- Designed specifically for automotive applications.

- Supports real-time communication between multiple Electronic Control Units (ECUs).

Wireless technologies such as Wi-Fi or Bluetooth are unsuitable because they introduce unpredictable delays and possible communication failures, which are unacceptable in safety-critical systems like automatic braking.

Therefore, CAN bus is the safest and most reliable communication method for this application

Answer:6

To measure the average temperature using only one sensor, I would place it at the center of the classroom, approximately half the height (b/2) from the floor. This location is likely to represent the average air temperature because it is away from walls, windows, doors, fans, air conditioners, and direct sunlight.

Poor placement can affect accuracy in several ways:

- Near a window: readings may be higher due to sunlight.

- Near an AC or fan: readings may be lower.

- Close to the ceiling: warm air rises, giving higher temperatures.

- Near the floor: readings may be lower than the room average.

- Near people or electronic equipment: body heat or device heat can increase the measured temperature.

Therefore, placing the sensor at the center of the room provides the most representative measurement.

Answer: 7

An analog sensor can perform better than a digital sensor in situations where:

- Very small changes in the measured quantity need to be detected.

- Fast response time is required because analog sensors provide continuous output.

- High-resolution ADCs are available to convert analog signals accurately.

- Cost is a major concern, as analog sensors are often cheaper.

- The application requires continuous monitoring rather than discrete digital values.

For example, an analog soil moisture sensor provides continuous voltage changes corresponding to moisture levels, allowing more detailed analysis than a simple digital wet/dry sensor.

Answer : 8

### Block Diagram

Sensor

│

▼

ESP32 Controller

│

▼

Wi-Fi Module

│

▼

Internet Router

│

▼

Cloud Server / Database

│

▼

Mobile App / Web Dashboard

│

▼

User

### Possible Failure Points and Solutions

| Failure Point | Possible Cause | Solution |
| --- | --- | --- |
| Sensor | Damage or incorrect calibration | Replace or recalibrate sensor |
| Wiring | Loose connections | Secure wiring and use quality connectors |
| ESP32 | Software crash | Enable watchdog timer and auto-restart |
| Wi-Fi | Network unavailable | Automatic reconnection and retry mechanism |
| Internet | ISP outage | Store data locally until connection returns |
| Cloud Server | Server downtime | Use backup server or redundant cloud service |
| Mobile App | App crash | Regular updates and error handling |

Answer: 9

If an attacker gains access to the IoT device through Wi-Fi, they may:

- Steal sensor data.

- Control actuators such as pumps or relays.

- Install malicious firmware.

- Change device settings.

- Use the device in a botnet or launch attacks on other systems.

### Security Improvements

- Use WPA2/WPA3 Wi-Fi encryption.

- Enable HTTPS/TLS encryption for communication.

- Use strong passwords and change default credentials.

- Regularly update firmware to fix security vulnerabilities.

- Implement user authentication and access control.

- Disable unused ports and services.

- Use firewalls and secure network configurations.

These practical measures greatly improve the security of the IoT system.

Answer: 10

Possible hardware and software conflicts include:

| Conflict | Solution |
| --- | --- |
| GPIO pin conflicts | Assign different GPIO pins to each device |
| Power supply overload | Use a separate power supply for the relay |
| Timing conflicts | Use non-blocking code with millis() instead of delay() |
| I²C address conflict | Ensure each I²C device has a unique address |
| ADC interference | Read analog sensors separately and average readings |
| Relay noise | Use opto-isolated relay modules and proper grounding |

Using FreeRTOS tasks on the ESP32 can also help different devices operate simultaneously without blocking each other.

Answer: 11

To prevent data loss:

- Continue collecting sensor data every minute.

- Store readings in local memory such as SPIFFS, LittleFS, or an SD card.

- Monitor internet connectivity periodically.

- When the connection is restored, upload all stored readings in chronological order.

- Confirm successful upload before deleting local copies.

- If uploading fails again, continue storing new data and retry later.

This ensures that no important sensor data is permanently lost.

Answer: 12

### Hardware Improvements

- Use industrial-grade sensors.

- Provide surge and over-voltage protection.

- Use high-quality capacitors and voltage regulators.

- Protect the device from dust and moisture using an IP-rated enclosure.

- Design efficient cooling and ventilation.

### Software Improvements

- Enable watchdog timers.

- Perform periodic self-diagnostics.

- Support OTA firmware updates.

- Store backup configurations.

- Implement automatic error recovery and reboot mechanisms.

These measures improve long-term reliability with minimal maintenance.

Answer: 14

Yes. An IoT system can be considered smart without using Artificial Intelligence.

For example, a smart irrigation system automatically turns the pump on when soil moisture falls below a preset threshold. This system is smart because it senses, processes, and acts automatically based on predefined rules.

An intelligent system goes one step further. It uses AI or machine learning to learn from past data and make predictions. For example, it may predict rainfall using weather forecasts and decide not to water the plants even if the soil moisture is low.

Difference:

| Smart System | Intelligent System |
| --- | --- |
| Rule-based | Learns from data |
| Fixed logic | Adaptive decisions |
| No learning | Uses AI/ML |
| Predictable | Can improve over time |

Therefore, every intelligent system is smart, but not every smart system is intelligent.

| Polling | Interrupt |
| --- | --- |
| CPU continuously checks devices | CPU responds only when an event occurs |
| Higher CPU usage | Lower CPU usage |
| Slower response | Faster response |
| Easier to implement | Slightly more complex |

Interrupts significantly improve IoT performance when:

- Detecting motion sensors.

- Receiving emergency signals.

- Handling push buttons.

- Monitoring fire or gas alarms.

- Measuring pulse sensors.

- Capturing ultrasonic echo signals.

Interrupts reduce unnecessary CPU activity and improve response time.

Answer (Smart Irrigation System): 15

### 1. Cost vs Performance

I selected the ESP32 instead of the Arduino UNO. Although the ESP32 costs slightly more, it provides built-in Wi-Fi, faster processing, and more memory.

### 2. Power Consumption vs Communication

Sending data every 5 seconds consumes more battery but provides real-time monitoring. This trade-off improves user experience.

### 3. Accuracy vs Cost

I used a capacitive soil moisture sensor instead of a resistive one. It is more expensive but provides better accuracy and longer lifespan.

These trade-offs were justified because they improve system reliability and performance.

Answer:16 step by step

1.Start

2.Initialize ESP32 and Sensors

3.Read Sensor Data

4. Wi-Fi Connected?

5. Yes

6.Upload Data to Cloud

7.Success?

7.7Yes                  7.2 No

8. Delete Local Copy     9.Save Data Locally

10.    Wait 1 Minute

11.  Repeat

### Components Used

- ESP32

- Soil moisture sensor

- Wi-Fi

- MQTT or HTTP protocol

- SPIFFS/LittleFS/SD Card

- Cloud database

- Mobile application

Answer:17   step by step

- Start

- Connect to Wi-Fi

3.Check for New Firmware

4.Update Available?

5. No

6.Run Existing Firmware

7.Yes

8.Download Firmware

9.Verify Integrity (Checksum)

10.Verification Successful?

11. No

12.Cancel Update

13.Yes

14.Install Firmware

15.Restart ESP32

16.Boot Successful?

17. Yes

18.Run New Firmware

19.No

20.Rollback to Previous Firmware

21.Restart Device

22.End.